# Non-Destructive Testing Campaign on a Steel Bridge through Acoustic Emission Sensors and Accelerometers

Acoustic Emission (AE) data:

The data are included in the Excel file “AE_PrimaryData.xlsx”. The location of the AE sensors on the bridge during the acquisition is reported in the image “AE_SensorsLocation.jpg”.

Hit data are generated by each AE channel independently and describe usually one or a series of discrete AE signals with an identifiable beginning and ending. The beginning of a hit is defined by its first threshold crossing, the end, by the absence of threshold crossings for a user-defined period of time, which is the ‘Duration Discrimination Time’. For each hit acquired by a DAQ channel, the following information are obtained:

- SetID

In the primary data file, each data set is automatically assigned a continuous and unique ID number (SetID).

- Time: Arrival Time

Time is the time measurement in seconds of the first threshold crossing of a burst signal. It starts off with value of 0 at the start of the measurement.

- Chan: Channel Number

Chan is the logical number of the channel that has produced the data set.

- Thr: Detection Threshold

Thr holds the value of the detection threshold in units of dBAE of the time when the data set was generated. A data set is generated either with the first threshold crossing (hit) or time triggered in case of status data sets.

- Amp: Burst Signal linear Peak Amplitude

Amp is the burst signal peak amplitude in μV.

- RiseT: Burst Signal Rise-time

RiseT is the burst signal rise time in units of μs and corresponds to the difference of the time of occurrence of the peak amplitude to the first threshold crossing.

- Dur: Burst Signal Duration

Dur is the duration of a burst signal in μs and corresponds to the time difference of last threshold crossing and first threshold crossing.

- Eny: Burst Signal Energy

Eny corresponds to the signal energy of the burst signal scaled to the arbitrary unit of eu. The signal energy is the sum of squared voltage samples (ui) starting with the sample of the first threshold crossing (i=1) up to the sample with which the status interval ends (i=N):

E=∆ti=1Nui2

Δ𝑡 refers to the time difference between two consecutive voltage samples.

- SS: Burst Signal Strength

SS corresponds to the signal strength of a burst signal in units of nVs. The signal strength is the sum of voltage samples (ui) starting with the sample of the first threshold crossing (i=1) up until the sample with which the Duration Discrimination Time (DDT) expired (i=N):

E=∆ti=1Nui

and Δ𝑡 refers to the time difference between two consecutive voltage samples.

- RMS: Background Noise

RMS is the last RMSS result assigned to a hit data set. The RMS Background noise is the root-mean square value of the noise on each channel during pauses of threshold crossings. It is derived from energy measurement. If no hits arrive, the energy is accumulated in time slices of 13ms. The energy rate per 13ms time interval is fed over a variable digital low-pass filter to achieve the user-selected time constant. The RMS is presented in μV.

- Counts: Ring down count

Counts is the number of positive threshold crossings up until the duration discrimination time expired.

- TRAI: Transient Recorder Index

The Transient Recorder Index (TRAI) is a unique number that identifies the transient recorder data belonging to a hit. A TRAI will be generated only if the transient recorder is triggered for recording.

Vibrational measurements through accelerometers

The data are included in the .txt file in the folder “Acceleration_Data”. The location of the accelerometers on the bridge during the acquisition is reported in the image “Accelerometers_Scheme.jpg”. For the different channels, Table 1 contains the name, description and the measurement unit, as well as the related sensor position.

Table 1 – Channel name, description, measurement unit and sensor position.

| Chan. Name | Chan. Description | Chan. Measurement unit | Sensor Position |
| --- | --- | --- | --- |
| Time | Progressive time | seconds | - |
| B12_8553 | CH0 : Midspan (x) ﻿ | m/s2 | C |
| B12_22827 | CH1 : ﻿Midspan (y) | m/s2 | C |
| 393C_2592 ﻿ | CH2 : Midspan (z) | m/s2 | C |
| B12_25065 | CH3 : ﻿West Quarter (y) | m/s2 | B |
| 393C_2593 ﻿ | CH4 : West Quarter (z) | m/s2 | B |
| B12_25066 ﻿ | CH5 : East Quarter (y) | m/s2 | D |
| 393C_2591 ﻿ | CH6 : East Quarter (z) | m/s2 | D |
| B12_14480 | CH7 : West Tendon (y) | ﻿m/s2 | A |
| B12_29072 ﻿ | CH8 : East Tendon (z) | m/s2 | E |
| 393C_2116 ﻿ | CH9 : West Quarter (x) | m/s2 | B |

Notes:

- From 2024/07/09 14:50:07 (14:18) to 2024/07/09 16:55:44 (16:23) files (no. 24):

A_2024_07_09_14_50_07.txt

A_2024_07_09_14_55_35.txt

A_2024_07_09_15_01_02.txt

A_2024_07_09_15_06_30.txt

A_2024_07_09_15_11_58.txt

A_2024_07_09_15_17_26.txt

A_2024_07_09_15_22_53.txt

A_2024_07_09_15_28_21.txt

A_2024_07_09_15_33_49.txt

A_2024_07_09_15_39_16.txt

A_2024_07_09_15_44_44.txt

A_2024_07_09_15_50_12.txt

A_2024_07_09_15_55_39.txt

A_2024_07_09_16_01_07.txt

A_2024_07_09_16_06_35.txt

A_2024_07_09_16_12_02.txt

A_2024_07_09_16_17_30.txt

A_2024_07_09_16_22_58.txt

A_2024_07_09_16_28_25.txt

A_2024_07_09_16_33_53.txt

A_2024_07_09_16_39_21.txt

A_2024_07_09_16_44_48.txt

A_2024_07_09_16_50_16.txt

A_2024_07_09_16_55_44.txt

Other Files

The file “Metadata.docx” contains the related metadata.

The file “WCEE2024-Paper_AcousticEmissions_v6” is the related paper.